# Brocade Virtual Fabrics and Configuring the logical switches

# The XISL is using LWL SFPs for the 10KM distance XISL will take care of the “trunking” of the fabrics on either sides

- Logical switch 1 and 5 ====>default fabric.

- Logical switch 2 and 6 ====>FID1.

- Logical switch 3 and 7 ====>FID15.

At last Base switch with fabric ID 8 is created for XISL communication.

1.Only Switches having same FID’s can communicate.

2.Only switch 1 and switch 5 have same FID128, they can communicate, but communication takes place through

base switch ie through FID8 by following all the protocols defined for particular logical switch.

3.We can also have dedicated ISL for specific logical switches between physical switches, when dedicated ISL

Is present the communication takes place through that ISL, if not communication happens through XISL which

is connected through base switches .

STEPS TO CREATE VIRTUAL SWITCHES

## Step 1: Enabling VF

This command is to check whether VF(Virtual fabric is on or off)

admin> fosconfig --show

FC Routing service:             disabled

iSCSI service:                  Service not supported on this Platform

iSNS client service:            Service not supported on this Platform

Virtual Fabric:                 disabled

Ethernet Switch Service:        Service not supported on this Platform

so first enable the feature. Warning though!! The switch will reboot immediately after hitting “Y”

This command is to turn-on VF(virtual fabric).

admin> fosconfig --enable vf

WARNING:  This is a disruptive operation that requires a reboot to take effect.

All EX ports will be disabled upon reboot.

Would you like to continue [Y/N]: Y

VF has been enabled.  Your system is being rebooted.

## Step 2: Create a base switch

Notice we’ve got a default fabric 128 after enabling VF. You can see in which FID context you currently are in the identifier in front of your username.

FID128:admin> lscfg --create 8 -base

Creation of a base switch requires that the proposed new base switch on this system be disabled.

Would you like to continue [y/n]?: y

About to create switch with fid=8. Please wait...

Logical Switch with FID (8) has been successfully created.

Logical Switch has been created with default configurations.

Please configure the Logical Switch with appropriate switch

and protocol settings before activating the Logical Switch.

Base switch with FID8 has been created. (ports are not added to base switch yet).

As you can notice the context is set to FID128 default.

FID 8 is created, so let’s assign some ports.
Lets also switch context to FID8 when configuring the ports for the base switch by

NOTE: Before adding ports to FID, please set context to FID which you want add the ports.

FID128:admin>setcontext 8

8 is FID number

Configure the port for the base switch. Port 6 for example, as shown in the figure above.

FID8:admin> lscfg --config 8 -port 6

This operation requires that the affected ports be disabled.

Would you like to continue [y/n]?: y

Making this configuration change.  Please wait...

Configuration change successful.

Please enable your ports/switch when you are ready to continue.

Now you can see context is changed to FID8 and port 6 is added/assigned to FID8.

Command to change the switch name:

1.First be in the context of the particular FID

FID8:admin>switchname baseswitch

“baseswitch” is user defined name.

## Step 3: Create the logical switches

The number of logical switches you can create depends on the platform you use. It’s mentioned in the guides. I’m working with a 5300, so i’m limited to 4 logical switches at time of writing this.

Besides the base switche and the default switch, i can create two more logical switches on the 5300. It does not really matter in which context you create the switches, as long you have permissions to do so

1.Now let’s Create FID1 and assign port 1 to it.

FID8:admin> lscfg --create 1

FID1 is created.

FID8:admin> setcontext 1

As we Context has to be set before adding ports to FID.

FID1:admin> switchname fab1-lsw1

Context changed to FID1 and switch named as “fab1-lsw1” , which is user defined name.

FID1:admin> lscfg --config 1 -port 2

Port 2 has been added to FID1.

2.Now lets create one more FID15(logical switch) and add port5 to it.

FID1:admin> lscfg --create 15

FID15 is created.

FID1:admin> setcontext 15

Context has to set before adding ports.

FID15:admin> switchname fab2-lsw-2

Here Context changed to FID15 and logical switch is named as “fab2-lsw-2”.

FID15:admin> lscfg –config 15 -port 5

Now the port 5 has been added to FID15.

FID15:admin> portcfgpersistentenable 5

This command is used to enable assigned ports to FID, 5 is the port number

Now we just created

1.Base switchbaseswitchFID8port 6.

2.Logical switchfab1-lsw1FID1port 2.

3.Logical switchfab2-lsw2FID15port 5.

As of now we just created logical switches in First physical switch same thing has to be done in second physical switch as well to form a fabric.

For example you can have different ports but use same FID for establishing communication.

FID1    port2,port7

fab1-lsw1

FID1         port2

fab1-lsw1

FID15   port 1,8

Fab2-lsw-2

FID15   port 5

Fab2-lsw-2

XISL

FID8      port 9

baseswitch

FID8      port 6

baseswitch

We can also have different port in same FID, still we can establish the communication

NOTE: XISL between port 6 and port 9 of base switches of two different switches must be a physical connection.

## Step 3: Connect the (X)ISL

When you physically connect the 5300 switches (port 6 on the first switch to port 8 on the second) fabric merges for fabrics with the same FIDs start to happen, just as they normally would when not using VF.
When done right, you’ll have 4 Fabrics on your switches: One default, one base, and two normal logical switches. Each one can be configured individually. The “fabricshow” command will output two switches in each fabric, depending on the cli context you’re in.

When you login using Web Tools GUI, you’ll be able to zone each fabric individually.

Important commands:

- lscfg –show : Lists all the FID’s in the physical switch.

- switchshow: lists all the ports present in the switch, depends which FID context we are in.

- lscfg –config -port 1: if we execute same command, ports will move back to default.

- lscfg –delete 15: this is command to delete the entire FID, ports move back to default FID

IMPORTANT NOTE: you have to be in default context (FID128) while executing or deleting the FID’s.

- fosconfig –disable vf : command to disable vf mode (Disruptive activity, requires a reboot).

- Fabricshow: