# zoZoning in Brocade.

Switch Port 1 – HBA1 – 20:00:00:11:0a:02:29:b1

Switch Port 2 – HBA2 – 20:00:00:11:0a:02:29:b2

Switch Port 12 – Storage Array port1 – 50:01:10:80:00:ad:33:e8

Switch Port 13 – Storage Array port2 – 50:02:10:80:00:ac:f5:54

### Basics of Zoning

Each device in a fiber channel fabric will have a unique Word Wide Name (WWN). WWN is a unique identifier which is burned into the hardware. Each vendor gets their own identifier. It is a 64-bit address. It is similar to an MAC address that you find in a NIC. There are two types of WWNs,

- Word Wide Node Name (WWNN)

- Word Wide Port Name (WWPN)

switch:admin> switchshow

### Brocade Switch Zoning Steps

Step 1: First assign alias names for WWPN’s of HBA(Hosts) and Storage array.

Alias names are created for easy understanding of admin.

Hostport1 is the name of WWPN 20:00:00:11:0a:02:29:b1.(HBA host).

->To check the alias name run.

Cmd: alishow “HostPort1” or alishow -i

Step 2: Now we are going to create two zones with two aliases

1.Here we created zone1 and assigned Hostport1 and storagePort1 to it.

2.Here we created zone2 and assigned Hostport2 and storagePort2 to it.

Step3 : Now create zone configurations and zones to it.

As we can see here the ports are already in the zone and we are assigning those zone to “AppServer” cfg

Next step is to keep cfg in enable state.

Step 4: Final step Enabling the cfg

| aliAdd | Add a member to a zone alias |
| --- | --- |
| aliCopy | Copy a zone alias |
| aliCreate | Create a zone alias |
| aliDelete | Delete a zone alias |
| aliRemove | Remove a member from a zone alias |
| aliRename | Rename a zone alias |
| aliShow | Print zone alias information |
| cfgAdd | Add a member to a configuration |
| cfgCopy | Copy a zone configuration |
| cfgCreate | Create a zone configuration |
| cfgDelete | Delete a zone configuration |
| cfgRemove | Remove a member from a configuration |
| cfgRename | Rename a zone configuration |
| cfgShow | Print zone configuration information |
| zoneAdd | Add a member to a zone |
| zoneCopy | Copy a zone |
| zoneCreate | Create a zone |
| zoneDelete | Delete a zone |
| zoneRemove | Remove a member from a zone |
| zoneRename | Rename a zone |
| zoneShow | Print zone information |
| cfgClear | Clear all zone configurations |
| cfgDisable | Disable a zone configuration |
| cfgEnable | Enable a zone configuration |
| cfgSave | Save zone configurations in flash |
| cfgSize | Print size details of zone database |
| cfgActvShow | Print effective zone configuration |
| cfgTransAbort | Abort zone configuration transaction |